blast-physics-js

A JavaScript Blast Physics Engine for the Mining Industry

Library Design Document

Brent Buffham

blastingapps.com  •  kirra-design.com

March 2026

Open-source npm package for blast vibration prediction, damage modelling,

detonation simulation, and flyrock analysis in the browser and Node.js.

# 1. Vision

blast-physics-js is a free, open-source npm library that provides blast engineering models for the mining industry, implemented in pure JavaScript with zero rendering dependencies. It is the physics companion to Kirra, extracting the analytical models that currently live inside Kirra's GPU shader pipeline into a standalone, testable, composable library that any mining engineer or software developer can use.

The library follows the same philosophy as trimesh-boolean: clean API, well-documented, community-oriented, published on npm with MIT licence.

## 1.1 Design Principles

- Zero rendering dependencies: no Three.js, no WebGL, no DOM. Pure computational library.

- Isomorphic: runs identically in browser (ES modules) and Node.js (CommonJS).

- SI units throughout: metres, kilograms, seconds, Pascals, m/s.

- Typed array output: all grid/field computations return Float32Array or Float64Array for direct GPU upload or binary serialisation.

- Model fidelity hierarchy: simple site-law PPV through to full Blair & Minchinton time-domain waveform superposition.

- Per-deck architecture: multi-deck holes, air gaps, and per-product VOD/density are first-class concepts inherited from Kirra's deck data model.

## 1.2 Target Users

- Blast engineers wanting PPV/damage predictions in browser-based or Node.js tools.

- Mining software developers integrating vibration/damage analytics into their platforms.

- Researchers exploring Heelan/Blair models without writing their own solver.

- Kirra itself: blast-physics-js becomes Kirra's upstream dependency for all analytical models.

# 2. Architecture Overview

The library is organised into five core domains, each mapping directly to a family of blast engineering computations. Every module exports pure functions and/or classes with no side effects.

| Domain | Modules | Description |
| --- | --- | --- |
| Vibration | PPV, ScaledHeelan, HeelanOriginal, BlairMinchinton | Peak particle velocity from site-law through full time-domain waveform superposition |
| Damage | HolmbergPersson, JointedRock | Near-field damage index, intact rock + Mohr-Coulomb joint failure |
| Pressure | BoreholePressure, PowderFactor | Borehole wall pressure attenuation, volumetric powder factor |
| Detonation | DetonationSimulator | Multi-primer front propagation, collision detection, non-linear Em computation |
| Flyrock | FlyrockModel | Ballistic trajectory shroud generation from Richards-Moore, Lundborg, McKenzie models |
| Movement | BlastMovement, VoxelGrid, DisplacementField | Physics-based blast throw simulation: voxelisation, Rapier3D dynamics, displacement vectors |

## 2.1 Package Structure

blast-physics-js/

src/

index.js                    // Main entry point, re-exports all public API

core/

ChargeColumn.js           // Charge column geometry + discretisation

DeckEntry.js              // Per-deck data structure (from Kirra DeckDataTexture)

RadiationPattern.js       // Blair/Heelan radiation patterns F1, F2, sfacp, sfacs

Waveform.js               // Blair pulse shape w(p) and derivatives

RockMass.js               // Rock properties, Vs derivation from Vp + Poisson

vibration/

PPV.js                    // Simple site-law PPV = K * SD^(-B)

PPVDeck.js                // Per-deck PPV with charge centroid evaluation

ScaledHeelan.js           // Blair non-linear superposition + radiation (RMS)

ScaledHeelanBlair.js      // Blair Lite: sfacp/sfacs patterns, primer-aware

HeelanOriginal.js         // First-principles Heelan 1953 via Blair 1996

BlairMinchinton.js        // Full time-domain coherent waveform superposition

damage/

HolmbergPerssonDamage.js  // H-P near-field damage index

JointedRockDamage.js      // Combined intact + joint failure

pressure/

BoreholePressure.js       // Pb = rho_e * VOD^2 / 8, attenuation P(R)

PowderFactor.js           // Volumetric PF = mass / capsule volume

detonation/

DetonationSimulator.js    // Multi-primer front propagation + collision

EmComputation.js          // Non-linear superposition Em values

flyrock/

FlyrockTrajectory.js      // Ballistic max-distance per hole

FlyrockShroud.js          // Heightfield grid + triangle mesh generation

movement/

VoxelGrid.js              // Voxelise rock mass within surface bounds

InitialVelocityField.js   // PPV-to-velocity conversion + direction

BlastMovementSimulator.js // Rapier3D WASM time-stepping simulation

DisplacementField.js      // Extract displacement vectors (SmartVectors)

PostBlastSurface.js       // Generate predicted post-blast topography

workers/

BlairHeavyWorker.js       // Web Worker for time-domain grid computation

FlyrockWorker.js          // Web Worker for shroud generation

test/

dist/

package.json

# 3. Core Data Structures

The library uses plain JavaScript objects for input data, aligned directly to Kirra's charging model and blast hole data structure. No class inheritance hierarchies; just typed shapes that are easy to serialise, pass to workers, and pack into GPU textures.

The design is driven by Kirra's four typed deck categories (INERT, COUPLED, DECOUPLED, SPACER) and the distinction between borehole diameter and charge diameter for decoupled charges.

## 3.1 HoleEntry

The blast hole geometry and physical properties. Aligned to Kirra's Blast Hole Management data structure. Every hole carries its borehole diameter, which is the physical drilled diameter used for borehole pressure calculations and as the default charge diameter for COUPLED decks.

// HoleEntry: blast hole geometry and properties

{

// Identity

entityName,              // Blast pattern name (e.g. 'Pattern_A')

holeID,                  // Unique hole identifier within entity

// Collar position (start point)

collarX, collarY, collarZ,   // World-space collar (m)

// Toe position (end point)

toeX, toeY, toeZ,            // World-space toe (m)

// Physical properties

holeDiamMm,              // Borehole diameter (mm) - default 115

holeType,                // 'Production', 'Presplit', 'Buffer', etc.

// Bench geometry (for flyrock/SDoB calculations)

benchHeight,             // Collar Z to grade Z (m)

subdrillLength,          // Grade to toe along hole vector (m)

// Timing

holeTime,                // Surface initiation time (ms)

// Measured data (optional)

measuredMass,            // Actual explosive mass loaded (kg)

measuredLength,          // Actual drilled length (m)

}

## 3.2 DeckEntry

The fundamental unit of blast data for physics models. Every vibration, damage, and pressure model operates on arrays of DeckEntry objects. Aligned to Kirra's prepareDeckDataTexture layout and the four typed deck categories from the Charging System.

The key distinction is between holeDiamMm (the physical borehole) and chargeDiamMm (the effective charge diameter). For COUPLED decks, chargeDiamMm equals holeDiamMm because the explosive fills the hole. For DECOUPLED decks, chargeDiamMm is the product's package diameter, which is smaller than the borehole, creating an air gap.

// DeckEntry: one deck within a blast hole

{

// Deck type (Kirra's four typed categories)

deckType,                // 'COUPLED' | 'DECOUPLED' | 'INERT' | 'SPACER'

// World-space geometry

topX, topY, topZ,        // Charge top position (m)

baseX, baseY, baseZ,     // Charge base position (m)

// Explosive properties (from Kirra product)

mass,                    // Explosive mass in this deck (kg, 0 for INERT/SPACER)

density,                 // Explosive density (kg/L i.e. g/cc, from product)

vod,                     // Velocity of detonation (m/s, from product)

productName,             // Product name (e.g. 'ANFO', 'Emulsion', 'Stemming')

// Diameters

holeDiamMm,              // Borehole diameter (mm) - from parent hole

chargeDiamMm,            // Effective charge diameter (mm):

//   COUPLED:   = holeDiamMm (full borehole contact)

//   DECOUPLED: = product diameter (smaller, air gap)

//   INERT:     = holeDiamMm

//   SPACER:    = product diameter or holeDiamMm

// Timing

timingMs,                // Total detonation time: surface + downhole delay (ms)

// Parent hole reference

holeIndex,               // Index into HoleEntry array

primerFraction,          // Primer position within deck:

//   0.0 = primer at deck top

//   1.0 = primer at deck base

}

Physics model usage of the two diameters:

- Borehole Pressure: Pb = rho_e * VOD^2 / 8 uses holeDiamMm for wall pressure; coupling ratio correction uses chargeDiamMm / holeDiamMm.

- Mass calculation: COUPLED mass = density * pi * (holeDiamMm/2000)^2 * length. DECOUPLED mass = density * pi * (chargeDiamMm/2000)^2 * length.

- Heelan/Scaled Heelan: borehole radius (holeDiamMm/2000) for amplitude scaling and frequency parameter omega = VOD / (2 * radius).

- Flyrock SDoB: contributing mass uses holeDiamMm for COUPLED, chargeDiamMm for DECOUPLED.

- INERT and SPACER decks carry mass = 0 and are excluded from vibration/damage models, but their depths matter for stemming length and SDoB calculations.

## 3.3 RockMass

Rock mass properties used across multiple models. Vs is derived from Vp and Poisson's ratio rather than being an independent parameter (following Blair's convention).

// RockMass properties

{

density,           // Rock density (kg/m3), default 2700

pWaveVelocity,     // P-wave velocity Vp (m/s), default 4500

poissonRatio,      // Poisson's ratio nu, default 0.25

// Derived: sWaveVelocity = Vp / sqrt(2(1-nu)/(1-2nu))

}

## 3.4 Computation Result Types

All spatial field computations return a GridResult object:

// GridResult: 2D scalar field

{

data,         // Float32Array, row-major [rows * cols]

rows, cols,   // Grid dimensions

minX, minY,   // World-space origin

cellX, cellY, // Cell spacing (m)

elevation,    // Z elevation of grid plane (m)

unit,         // String: 'mm/s', 'MPa', 'DI', 'kg/m3', 'm/kg^(1/3)'

model,        // String: model name

}

## 3.5 Deck Type Semantics

The four deck types from Kirra's charging system map to specific physics behaviours:

| Deck Type | Charge Diameter | Mass Contribution | Physics Models |
| --- | --- | --- | --- |
| COUPLED | = holeDiamMm | Full: density * pi * (holeDiam/2)^2 * L | PPV, Heelan, Damage, Pressure, PF, SDoB, Flyrock |
| DECOUPLED | = product diameter | Reduced: density * pi * (prodDiam/2)^2 * L | PPV, Heelan, Damage, Pressure (with coupling ratio), Flyrock |
| INERT | = holeDiamMm | mass = 0 (non-explosive) | Stemming length for SDoB and Flyrock only |
| SPACER | = product or hole | mass = 0 (non-explosive) | Air deck gap geometry only |

# 4. Vibration Models

The vibration module provides a hierarchy of five models with increasing fidelity and computational cost, extracted directly from Kirra's shader/worker implementations.

## 4.1 PPV — Simple Site Law

The simplest model. Point-source evaluation at charge centroid using the standard scaled distance law:

PPV = K * (D / Q^e)^(-B)

Where: K = site constant, B = site exponent, e = charge exponent (0.5 for square-root scaling), D = distance (m), Q = charge mass (kg). Evaluates at charge top, centroid, and base positions; returns the maximum. Supports timing window for MIC (Maximum Instantaneous Charge) computation.

Computational complexity: O(n) per evaluation point, where n = number of decks.

## 4.2 PPVDeck — Per-Deck Site Law

Evaluates PPV at three positions along each deck (top, centre, base) using each deck's individual mass. Multi-deck holes produce separate influence zones; air gaps are naturally excluded. Identical physics to PPV but with deck-level granularity.

## 4.3 ScaledHeelan — Blair Non-Linear Superposition (RMS)

Blair & Minchinton (2006) Scaled Heelan model with incoherent (RMS) energy summation. Each deck is subdivided into elemsPerDeck sub-elements. Blair's (2008) non-linear charge superposition computes Em per element:

Em = [m * w_e]^A - [(m-1) * w_e]^A

PPV_element = K * Em * R^(-B) * F(phi)

Radiation patterns use Heelan's F1(phi) = sin(2phi)*cos(phi) and F2(phi) = sin(phi)*cos(2phi). Viscoelastic attenuation via quality factors Qp, Qs. This is the GPU-friendly model: no time loop, suitable for real-time interactive use.

Computational complexity: O(n * M) per point, where M = elements per deck.

## 4.4 ScaledHeelanBlair — Blair Lite (Blair Radiation Patterns)

Same RMS energy summation as ScaledHeelan but with Blair's improved radiation patterns:

sfacp = 1 - 2*(Vs/Vp)^2 * cos^2(phi)     // P-wave: non-zero on axis

sfacs = sin(2*phi)                          // SV-wave with fud=1.2 regularisation

Key improvements: Vs derived from Vp + Poisson's ratio (not independent parameter). On-axis PPV is non-zero. Primer-aware element ordering (distance from primer outward). Near-axial fud = 1.2 regularisation prevents SV collapse.

## 4.5 BlairMinchinton — Full Time-Domain Waveform Superposition

The most physically faithful model. Synthesises actual P-wave and SV-wave waveforms at each observation point, superposes them coherently across all elements and decks, and extracts the true peak vector particle velocity including constructive/destructive interference.

Waveform pulse shape (Blair 1996):

w(p) = (p^N - 2N*p^(N-1) + N(N-1)*p^(N-2)) * exp(-p)

where p = bandwidth * (t - t_arrival)

This is computationally expensive: O(n * M * T) per point where T = number of time steps. The library provides both a direct function (for single-point evaluation) and a BlairHeavyWorker for parallel grid computation via Web Workers with row-strip distribution.

Optimisations carried over from blairHeavyWorker.js: per-pixel pre-computation hoists all transcendentals out of the time loop; iterative multiply replaces Math.pow; waveform tail cutoff; squared-magnitude tracking defers sqrt; Float64Array typed arrays for cache-friendly access.

# 5. Damage Models

## 5.1 HolmbergPerssonDamage

Holmberg-Persson near-field damage model using per-deck sub-element integration. Each deck is subdivided into elements; PPV contributions are summed incoherently (RMS):

PPV_element = K_hp * (q * dL)^alpha / R^beta

PPV_deck = sqrt(sum(PPV_element^2))

DamageIndex = peakPPV / PPV_critical

Values > 1.0 indicate damage/failure. The damage index is clamped to [0, 1].

## 5.2 JointedRockDamage

Combines intact rock fracture with Mohr-Coulomb joint-controlled failure. Converts PPV to dynamic stress, then evaluates both mechanisms:

sigma_d = rho_rock * Vp * PPV        // Dynamic stress (Pa)

FR_rock = sigma_d / sigma_t            // Intact rock fracture ratio

FR_joint = tau / (c + mu * sigma_n)    // Joint failure ratio

DamageRatio = max(FR_rock, FR_joint)

# 6. Pressure & Energy Models

## 6.1 BoreholePressure

Borehole wall pressure with geometric attenuation along each deck's charge segment:

Pb = rho_e * VOD^2 / 8               // Wall pressure (Pa)

P(R) = Pb * (a / R)^alpha             // Attenuation

Uses distToSegment for smooth contours (no discrete element stepping). Per-deck VOD and density from charging data.

## 6.2 PowderFactor

Volumetric powder factor: distance R from observation point to the nearest point on each deck's charge segment, then:

PF = deckMass / ((4/3) * pi * R^3)    // kg/m3

# 7. Detonation Simulation

## 7.1 DetonationSimulator

Simulates detonation front propagation for charge columns with one or more primers. Uses Blair (2008) non-linear superposition generalised for arbitrary primer configurations.

Key capabilities:

- Multiple primers per column with independent fire times.

- Detonation front collision detection between opposing fronts from adjacent primers.

- Blocked-front logic: elements behind a collision point are not reached by that front.

- Output: per-element detonation time and Em (non-linear superposition equivalent mass).

## 7.2 EmComputation

Blair (2008) generalised cumulative mass approach:

Em_group = cumulativeMass_after^A - cumulativeMass_before^A

Em_per_element = Em_group / groupSize

Invariant: Σ Em = totalMass^A. Simultaneous detonations (within 0.01ms tolerance) are grouped and split equally.

# 8. Flyrock Analysis

## 8.1 FlyrockTrajectory

Computes maximum flyrock distance for each hole using ballistic trajectory equations. Three algorithm options: Richards-Moore, Lundborg, McKenzie. Factor of Safety applied to maximum velocity.

## 8.2 FlyrockShroud

Generates a 3D heightfield surface (triangle mesh) representing the flyrock exclusion zone envelope across the entire blast pattern. Uses the same grid-based approach as the existing flyrockWorker.js:

- Compute grid bounding box from all hole max-distances with padding.

- For each grid cell, evaluate the maximum altitude from all holes' ballistic envelopes.

- Triangulate the heightfield grid, applying end-angle culling for steep edges.

- Output: array of points + triangles, ready for Three.js or any mesh renderer.

# 9. Blast Movement (Throw Model)

Blast movement prediction is the highest-value module in the library. Understanding where the rock mass moves during a blast is critical to separating ore and waste effectively. This module provides an open-source alternative to proprietary solutions like Orica's OREPro 3D Predict, which uses physics-based algorithms to predict movement dynamics throughout the entire blast volume.

## 9.1 Concept

The key insight is that the vibration models from Phases 1–4 provide the initial conditions for the movement simulation. The Scaled Heelan or Blair model computes a PPV field that represents the energy distribution. The timing sequence determines when each region of rock is activated and in which direction the impulse acts (toward the free face). A physics engine then simulates the dynamics: gravity, friction, collisions, and muckpile formation.

The workflow mirrors OREPro 3D Predict's approach:

- Input: pre-blast surface(s), blast block boundaries, pit shell, hole geometry, charging, and timing — all from Kirra's CAP file export.

- Voxelise: discretise the rock mass within the surface bounds into a 3D grid of rigid bodies.

- Initial velocity: compute per-voxel velocity vectors from the blast energy field. Direction is toward the nearest free face, modulated by the Heelan/Blair radiation pattern. Magnitude is derived from PPV-to-velocity conversion.

- Simulate: step forward in time using a WASM physics engine (Rapier3D). Each voxel is a rigid body with gravity, inter-body friction, and boundary collisions against pit shell surfaces.

- Output: displacement vectors (initial → final position for each voxel), predicted post-blast surface mesh, and swell factor distribution.

## 9.2 Surface Naming Convention

Kirra's CAP file contains named surfaces. The blast movement module uses naming conventions to identify surface roles:

| Surface Name | Role | Physics Treatment |
| --- | --- | --- |
| blast_block / blast_loft | Rock mass to be blasted | Voxelised into rigid bodies — receives initial velocity |
| pit_shell / highwall | Pit boundary (static) | Static collider — voxels bounce off this |
| floor / grade | Bench floor surface | Static collider — ground plane for muckpile |
| free_face / face | Exposed face of bench | Direction reference — throw direction toward face |
| toe_bund / catchment | Toe catchment berm | Static collider — muckpile containment |

## 9.3 Voxelisation

The rock mass within the blast block boundary is discretised into a regular 3D grid of voxels. Each voxel becomes a rigid body in the physics simulation.

- Voxel size is configurable (default: 1m cubes for production blasts, 0.5m for presplit/buffer).

- Only voxels whose centre falls inside the blast block surface AND above the floor surface are included.

- Each voxel stores: position, mass (rock density × volume), and a reference to the nearest blast hole(s).

- Typical counts: a 50m × 80m × 12m bench at 1m resolution = ~48,000 voxels. Rapier3D WASM handles this scale.

## 9.4 Initial Velocity Computation

The vibration models provide the energy field. Converting PPV to initial velocity uses the impulse-momentum relationship:

// Per-voxel initial velocity

V_magnitude = PPV_peak * velocityScaleFactor    // PPV from Scaled Heelan/Blair

V_direction = normalize(toFreeFace)              // Toward nearest free face

V_direction.z += heaveComponent                  // Vertical heave from gas pressure

// Timing activation: voxel only moves when nearest hole fires

activationTime = nearestHole.timingMs + travelDelay

The velocityScaleFactor is a calibration parameter that converts PPV (mm/s at a point) to bulk rock velocity (m/s of the mass). This is the primary tuning knob — analogous to OREPro 3D's SmartVector calibration from historical blast data.

Direction computation accounts for multiple free faces (corner shots), burden relief angle, and the blast timing sequence which creates differential movement (earlier-firing rows push into later-firing rows).

## 9.5 Physics Engine Integration

The simulation uses Rapier3D (Rust → WASM) for rigid body dynamics. Rapier was chosen over Jolt and cannon-es for:

- WASM performance: near-native speed for 10,000–50,000 rigid bodies.

- Cross-platform determinism: identical results across browsers and machines.

- Snapshot/restore: save and reload simulation state for replay.

- Active maintenance: Dimforge's 2025 focus was specifically on WASM/JS performance.

Simulation parameters:

- Gravity: 9.81 m/s² downward.

- Inter-voxel friction: coefficient 0.5–0.8 (calibrated from muckpile angle of repose).

- Coefficient of restitution: 0.1–0.3 (rock-on-rock, low bounce).

- Swell factor: 1.3–1.5 (voxels expand slightly post-blast to account for fragmentation swell).

- Time step: 1/60s, simulate 5–15 seconds of real time (typical blast duration + settling).

- Damping: angular and linear damping to prevent unrealistic spinning/bouncing.

## 9.6 Output: Displacement Vectors

After the simulation settles, the displacement field is extracted:

// DisplacementVector per voxel

{

startX, startY, startZ,    // Pre-blast centroid position

endX, endY, endZ,          // Post-blast settled position

dx, dy, dz,                // Displacement vector (end - start)

magnitude,                 // Total displacement (m)

activationTime,            // When this voxel started moving (ms)

}

These vectors can be applied to an in-situ block model to produce a swelled post-blast grade control model — the same output concept as OREPro 3D's SmartVectors.

## 9.7 Output: Post-Blast Surface

The final voxel positions are used to generate a predicted post-blast muckpile surface:

- Extract the top-most voxel at each XY grid position → heightfield.

- Triangulate the heightfield → triangle mesh.

- Compare to pre-blast surface → swell/heave/lateral movement maps.

- Export as Surpac DTM/STR or DXF for mine planning software.

## 9.8 Calibration

The model accuracy depends on calibrating the velocityScaleFactor and friction parameters against real blast data. Two approaches:

Reactive calibration: Given pre-blast and post-blast survey surfaces for a historical blast, run the simulation with varying parameters until the predicted post-blast surface matches the actual survey. This is analogous to OREPro 3D's reactive workflow. With 10–20 calibrated blasts from a site, the parameters converge for that rock mass.

ML-assisted calibration: Given a database of pre/post surface pairs + loading data (the 500–1000 blasts you mentioned), train a regression model to predict the velocityScaleFactor and friction coefficients as a function of rock type, explosive type, powder factor, and SDoB. This is the long-term path to site-independent prediction.

# 10. API Design

The library exposes two levels of API: high-level convenience functions for common tasks, and low-level model classes for full control.

## 9.1 High-Level API

import { computePPV, computeScaledHeelan, computeDamage } from 'blast-physics-js';

// Single-point PPV evaluation

const ppv = computePPV(

{ x: 100, y: 200, z: 0 },     // observation point

deckEntries,                     // array of DeckEntry

{ K: 1140, B: 1.6, e: 0.5 }    // site constants

);

// Returns: number (mm/s)

// Grid computation (returns GridResult)

const grid = computeScaledHeelan(

deckEntries, holeEntries,

{ minX: 0, minY: 0, maxX: 200, maxY: 200, cellSize: 1.0, elevation: 0 },

{ K: 1140, B: 1.6, chargeExponent: 0.5, elemsPerDeck: 12, ... }

);

// Returns: GridResult { data: Float32Array, rows, cols, ... }

## 9.2 Low-Level API

import { ScaledHeelanModel, BlairMinchintonModel } from 'blast-physics-js';

const model = new ScaledHeelanModel({

K: 1140, B: 1.6, chargeExponent: 0.5,

elemsPerDeck: 20, pWaveVelocity: 4500, poissonRatio: 0.25

});

// Evaluate at a single point

const ppv = model.evaluate(observationPoint, deckEntries, holeEntries);

// Compute a grid

const result = model.computeGrid(deckEntries, holeEntries, gridParams);

## 9.3 Worker API

import { createBlairWorker } from 'blast-physics-js/workers';

const worker = createBlairWorker();

worker.compute({

deckEntries, holeEntries,

gridParams: { minX, minY, elevation, ... },

modelParams: { K: 700, B: 1.5, bandwidth: 10000, ... },

startRow: 0, endRow: 100

}).then(strip => {

// strip: Float32Array

});

# 11. Extraction Plan from Kirra

The library extracts physics code from Kirra's existing codebase. This table maps Kirra source files to blast-physics-js modules:

| Kirra Source | Library Module | Extraction |
| --- | --- | --- |
| PPVModel.js | vibration/PPV.js | GLSL → JS |
| PPVDeckModel.js | vibration/PPVDeck.js | GLSL → JS |
| ScaledHeelanModel.js | vibration/ScaledHeelan.js | GLSL → JS |
| ScaledHeelanNinetyPCTModel.js | vibration/ScaledHeelanBlair.js | GLSL → JS |
| HeelanOriginalModel.js | vibration/HeelanOriginal.js | GLSL → JS |
| BlairMinchintonModel.js | vibration/BlairMinchinton.js | GLSL → JS |
| blairHeavyWorker.js | workers/BlairHeavyWorker.js | Direct port |
| NonLinearDamageModel.js | damage/HolmbergPerssonDamage.js | GLSL → JS |
| JointedRockDamageModel.js | damage/JointedRockDamage.js | GLSL → JS |
| PressureModel.js | pressure/BoreholePressure.js | GLSL → JS |
| PowderFactorModel.js | pressure/PowderFactor.js | GLSL → JS |
| DetonationSimulator.js | detonation/DetonationSimulator.js | Direct port |
| ElementDataPacker.js | (Kirra integration layer) | Not extracted |
| flyrockWorker.js | flyrock/FlyrockShroud.js | Direct port |

The extraction strategy is 'GLSL to JS' for shader-based models: the fragment shader's per-pixel computation logic is translated to equivalent JavaScript functions, stripping all GPU texture sampling, colour mapping, and rendering concerns. The underlying physics (site laws, radiation patterns, superposition) translates 1:1.

Three files remain in Kirra's integration layer and are NOT extracted: BlastAnalyticsShader.js (Three.js orchestrator), ElementDataPacker.js (GPU DataTexture packing), and PowderFactorModel.prepareDeckDataTexture() (Kirra-specific hole-to-deck data transform). These are consumers of blast-physics-js, not part of it.

# 12. Implementation Phases

## Phase 1: Core + Simple Vibration

- Set up npm package: blast-physics-js, MIT licence, Vite/Rollup build, ESM + CJS output.

- Core data structures: DeckEntry, HoleEntry, RockMass, GridResult.

- vibration/PPV.js: single-point and grid computation.

- vibration/PPVDeck.js: per-deck evaluation.

- Test suite with known-answer tests from Kirra's existing output.

- npm publish v0.1.0.

## Phase 2: Scaled Heelan + Damage

- core/RadiationPattern.js: F1, F2 (Heelan) and sfacp, sfacs (Blair).

- vibration/ScaledHeelan.js: Blair non-linear superposition with RMS summation.

- vibration/ScaledHeelanBlair.js: Blair Lite with improved radiation patterns.

- damage/HolmbergPerssonDamage.js and damage/JointedRockDamage.js.

- Validation against Kirra GPU shader output for identical inputs.

- npm publish v0.2.0.

## Phase 3: Blair Time-Domain + Workers

- core/Waveform.js: Blair pulse shape w(p) and iterative evaluation.

- vibration/HeelanOriginal.js: first-principles Heelan 1953 model.

- vibration/BlairMinchinton.js: full time-domain coherent superposition.

- workers/BlairHeavyWorker.js: row-strip parallelism for grid computation.

- Validation against blairHeavyWorker.js existing output.

- npm publish v0.3.0.

## Phase 4: Detonation + Flyrock + Pressure

- detonation/DetonationSimulator.js: multi-primer front propagation.

- detonation/EmComputation.js: non-linear Em values.

- flyrock/FlyrockTrajectory.js and flyrock/FlyrockShroud.js.

- pressure/BoreholePressure.js and pressure/PowderFactor.js.

- Full documentation, API reference, Buy Me a Coffee writeup.

- npm publish v1.0.0.

## Phase 5: Blast Movement (Throw Model)

Physics-based blast movement prediction inspired by OREPro 3D Predict. Uses blast-physics-js vibration models for initial conditions and a WASM physics engine for dynamics simulation.

- movement/VoxelGrid.js: voxelise rock mass within blast block and pit shell surface bounds.

- movement/InitialVelocityField.js: compute per-voxel velocity vectors from Scaled Heelan PPV field, timing sequence, and hole geometry (direction toward free face, magnitude from PPV-to-velocity conversion).

- movement/BlastMovementSimulator.js: time-stepping simulation using Rapier3D WASM physics engine — rigid body dynamics with gravity, inter-block friction, surface collisions, and swell factor.

- movement/DisplacementField.js: extract displacement vectors (SmartVectors equivalent) from initial to final voxel positions. Output as vector field for block model transformation.

- movement/PostBlastSurface.js: generate predicted post-blast topography mesh from final voxel positions.

- Integration with Kirra CAP file import for surfaces (blast block, pit shell) and hole/charging data.

- Web Worker parallelism for voxel computation and physics stepping.

- npm publish v2.0.0.

# 13. Testing Strategy

Every model is validated against Kirra's existing GPU shader and Web Worker output using snapshot testing:

- Generate a reference blast pattern (10 holes, known geometry + charging).

- Run each Kirra shader/worker model and capture output grids as JSON snapshots.

- blast-physics-js tests compute the same grids and assert agreement within tolerance (< 1% RMS error for analytical models, < 5% for time-domain due to stride/resolution differences).

- Unit tests for core functions: radiation patterns at known angles, waveform pulse shape at known p values, Em invariant sum, detonation collision detection.

## 12.1 Performance Benchmarks

Tracked per release:

- PPV grid 200x200 at 1m resolution: target < 50ms.

- ScaledHeelan grid 200x200 with 100 decks, 12 elements: target < 500ms.

- BlairMinchinton grid 100x100 with 50 decks, 12 elements: target < 10s (single worker).

- FlyrockShroud for 200 holes: target < 2s.

- BlastMovement 50m×80m×12m bench at 1m voxels (48k bodies): target < 60s for 10s simulation.

# 14. References

- Blair, D.P. & Minchinton, A. (1996). On the damage zone surrounding a single blasthole. Fragblast-5, Montreal.

- Blair, D.P. & Minchinton, A. (2006). Near-field blast vibration models. Fragblast-8, Santiago.

- Blair, D.P. (2008). Non-linear superposition models of blast vibration. Int. J. Rock Mech. Min. Sci. 45, 235–247.

- Blair, D.P. (2015). Wall control blasting. Fragblast 11, Sydney.

- Heelan, P.A. (1953). Radiation from a cylindrical source of finite length. Geophysics 18, 685–696.

- Holmberg, R. & Persson, P.A. (1979). Design of tunnel perimeter blasthole patterns to prevent rock damage. Tunnelling '79, London.

- Chiappetta, R.F. & Treleven, J.P. (1997). Scaled Depth of Burial concept for flyrock risk assessment.

- Richards, A.B. & Moore, A.J. (2004). Flyrock control — by chance or design. Proc. 30th ISEE Conf.

- McKenzie, C. (2009). Flyrock range and fragment size prediction. McKenzie (2022) validation.

- Orica OREPro 3D / OREPro 3D Predict. SmartVector blast movement technology. orica.com/orepro3d.

- Rapier Physics Engine (Dimforge). rapier.rs — Rust/WASM rigid body dynamics.